|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | H.12 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  |

SERIES H: TRANSMISSION OF NON-TELEPHONE SIGNALS

Characteristics of transmission channels used for other than telephone purposes

Characteristics of telephone-type leased circuits

ITU-T  Recommendation  H.12

Extract from  Red Book  Fascicle III.4 (1984)

NOTES

1	ITU-T Recommendation H.12 was published in Fascicle III.4 of the Red Book. This file is an extract from the Red Book. While the presentation and layout of the text might be slightly different from the Red Book version, the contents of the file are identical to the Red Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Ó  ITU  1984, 1988, 1993, 1996

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation H.12

Fascicle III.4 – Rec. H.12

CHARACTERISTICS OF TELEPHONE-TYPE LEASED CIRCUITS

(Mar del Plata, 1968; amended at Geneva, 1972, 1976 and 1980)

# 1	Ordinary telephone-type circuits

Note 1 – The application of § 1 of this Recommendation to multiterminal leased circuits is intended only for radial networks in which these specifications are to be met between a designated central station and each of the outlying stations. It does not apply to multiterminal conference networks between any two stations.

Note 2 – The contents of § 1 correspond to part of Recommendation M.1040 [2].

## 1.1	Scope of § 1 of the Recommendation and constitution of a leased circuit

Paragraph 1 of this Recommendation details the characteristics of international leased circuits for telephony and other purposes that do not require the use of special-quality leased circuits conforming to §§ 2 or 3. These circuits are set up as described in Recommendation M.1050 [1].

## 1.2	Characteristics

### 1.2.1	Nominal overall loss

Because of the differing nominal level at renters’ premises due to the various national practices, it is not normally possible to predict the nominal overall loss of the circuit at the reference frequency. Only exceptionally can a predetermined specified nominal overall loss at the reference frequency between renters’ installations be offered and then only after prior consultation among the Administrations concerned.

For 4-wire circuits, the receiving relative level at the renters’ premises should not be lower than –15 dBr. If a mean sending signal power of –15 dBm0 is assumed, the resulting minimum received power (–30 dBm) is sufficient for telephony and the other purposes for which circuits to this Recommendation are intended. Should these circuits be used for other purposes, higher receiving relative levels may be required in some circumstances.

It should be noted that the overall loss in each direction of transmission may not have the same value.

### 1.2.2	Loss/frequency distortion

The limits for the overall loss relative to that at 800 Hz for the circuit between renters’ installations are given in Figure 1/H.12.

### 1.2.3	Random circuit noise

The nominal level of the psophometric noise power at a renter’s premises depends upon the actual constitution of the circuit, in particular upon the length of frequency division multiplex carrier systems in the circuit. The provisional limit for leased circuits of distances greater than 10 000 km is –38 dBm0p. However, circuits of shorter length will have substantially less random noise (see also Annex A).

FIGURE 1/H.12

# 2	Special quality leased circuits with basic bandwidth conditioning1), 2)

## 2.1	Scope of § 2

Paragraph 2 of this Recommendation deals with leased circuits for uses other than telephony – for example, data transmission.

The characteristics of §§ 2 and 3 are intended to ensure the provision of a circuit which will meet the requirements of digital transmission rates higher than those possible on a normal telephone-type circuit3). § 2 is primarily intended for use with modems that contain equalizers.

## 2.2	Characteristics4)

Note – Except for the bandwidth characteristics (loss/frequency and group-delay distortions), the characteristics of § 2 are common to both types of “special quality leased circuits”, that is, to §§ 2 and 3 of this Recommendation.

### 2.2.1	Nominal overall loss

Because of the differing nominal level at renters’ premises due to the various national practices, it is not normally possible to predict the nominal overall loss of the circuit at the reference frequency. Only exceptionally can a predetermined specified nominal overall loss at the reference frequency between renters’ installations be offered and then only after prior consultation among the Administrations concerned.

For 4-wire circuits, the value of the receiving relative level at the renters’ premises should not be lower than _13 dBr.

For circuits intended to be used for data transmission using modems to V-series Recommendations, higher receiving relative levels may be required in some circumstances.

It should be noted that the overall loss in each direction of transmission may not have the same value.

### 2.2.2	Loss/frequency distortion

The limits for the overall loss relative to that at 800 Hz for the circuit between renters’ installations are given in Figure 2/H.12.

FIGURE 2/H.12

### 2.2.3	Group-delay distortion5)

The limits that apply to group-delay distortion are given in Figure 3/H.12 in which the limiting values over the frequency band are expressed as values relative to the minimum measured group delay.

### 2.2.4	Variation of overall loss with time

#### 2.2.4.1	Amplitude hits

Where the circuit is to be used for data transmission using modems employing amplitude modulation techniques, for example modems to Recommendation V.29, amplitude hits may result in data errors. Using an instrument complying with Recommendation O.95, the number of amplitude hits greater than  2 dB should not exceed 10 in any 15-minute measuring period. (The value of  2 dB and the number of amplitude hits are provisional and subject to further study.)

FIGURE 3/H.12

#### 2.4.4.2	Other variations

For all circuits, variations with time of the overall loss at 800 Hz (including daily and seasonal variations but excluding amplitude hits) should be as small as possible but should not exceed ± 4 dB.

### 2.2.5	Random circuit noise

The nominal level of the psophometric noise power at the renter’s premises depends upon the actual constitution of the circuit, in particular upon the length of frequency division multiplex carrier systems in the circuit. The provisional limit for special quality leased circuits of distances greater than 10 000 km is –38 dBm0p. However, circuits of shorter length will have substantially less random noise (see also Annex A).

Note – In the case of multiterminal circuits, the length to be taken into consideration is the sum of the lengths of the various sections of the link.

### 2.2.6	Impulsive noise

Impulsive noise should be measured with an instrument complying with Recommendation O.71 [4].

The number of impulsive noise peaks exceeding –21 dBm0 should not be more than 18 in 15 minutes.

### 2.2.7	Phase jitter

The value of phase jitter measured at a renter’s premises depends upon the actual constitution of the circuit (for example, upon the number of modulation equipments involved). It is expected that any measurement of phase jitter using an instrument complying with Recommendation O.91 [5] will not normally exceed 10° peak-to-peak. However, for circuits of necessarily complex constitution and where 10° peak-to-peak cannot be met, a limit of up to 15° peak-to-peak is permitted.

### 2.2.8	Quantizing noise (quantizing distortion)

If any circuit section is routed over a PCM system, the signal will be accompanied by quantizing noise. The minimum ratio of signal-to-quantizing noise normally expected is 22 dB.

### 2.2.9	Single tone interference

The level of single tone interference in the band 300-3400 Hz shall not exceed a value which is 3 dB below the circuit noise objective indicated in Figure A-1/H.12.

### 2.2.10	Frequency error

The frequency error introduced by the circuit must not exceed  5 Hz. It is to be expected that in practice the error will be within closer limits than these.

### 2.2.11	Harmonic distortion

When a 700-Hz test frequency of –13 dBm0 is injected at the transmit end of a point-to-point circuit, the level of any individual harmonic frequency at the receiving end shall provisionally be at least 25 dB below the received level of the fundamental frequency.

# 3	Special quality leased circuits with special bandwidth conditioning6), 7)

## 3.1	Scope of § 3

Paragraph 3 of this Recommendation deals with leased circuits for uses other than telephony – for example, data transmission.

The characteristics of §§ 2 and 3 of this Recommendation are intended to ensure the provision of a circuit which will meet the requirements of digital transmission rates higher than those possible on a normal telephone-type circuit. § 3 is primarily intended for use with modems that do not contain equalizers.

## 3.2	Characteristics

Note – As discussed in § 2, except for the bandwidth characteristics (loss/frequency and group-delay distortions), the characteristics of § 2 are common to both types of “special quality leased circuits”. Thus only the loss/frequency distortion and group-delay distortion will be described in this section.

### 3.2.1	Nominal overall loss

See § 2.2.1.

### 3.2.2	Loss/frequency distortion

The limits for the overall loss relative to that at 800 Hz for the circuit between renters’ installations are given in Figure 4/H.12.

### 3.2.3	Group-delay distortion

The limits that apply to group-delay distortion are given in Figure 5/H.12 in which the limiting values over the frequency band are expressed as values relative to the minimum measured group delay.

The other characteristics correspond to these of §§ 2.2.4 to 2.2.11 of this Recommendation (see the Note to § 2.2).

FIGURE 4/H.12

FIGURE 5/H.12

ANNEX A

(to Recommendation H.12)

Random circuit noise

Figure A-1/H.12 displays random noise versus length and is presented as a guide to the random noise performance which may be found on an international leased circuit.

FIGURE A-1/H.12

Note – At the present time the section of the circuit provided by satellite (between earth stations) contributes approximately 10 000 pW0p (–50 dBm0p) of noise. Therefore, for the purpose of determining maintenance limits for noise measurements on leased circuits, the length of a circuit section routed on a communications satellite may be considered to be equivalent to 1000 km in Figure A-1/H.12.

References

[1]	CCITT Recommendation Lining up an international point-to-point leased circuit, Vol. IV, Rec. M.1050.

[2]	CCITT Recommendation Characteristics of ordinary quality international leased circuits, Vol. IV, Rec. M.1040.

[3]	CCITT Recommendation Characteristics of special quality international leased circuits with basic bandwidth conditioning, Vol. IV, Rec. M.1025.

[4]	CCITT Recommendation Specification for an impulsive noise measuring instrument for telephone-type circuits, Vol. IV, Rec. O.71.

[5]	CCITT Recommendation Essential clauses for an instrument to measure phase jitter on telephone circuits, Vol. IV, Rec. O.91.

[6]	CCITT Recommendation Characteristics of special quality international leased circuits with special bandwidth conditioning, Vol. IV, Rec. M.1020.

1	)	The application of § 2 of this Recommendation to multiterminal leased circuits is intended only for radial networks in which these specifications are to be met between a designated central station and each of the outlying stations. It does not apply to multiterminal conference networks between any two stations.

2)	The contents of § 2 correspond to part of Recommendation M.1025 [3].

3	)	In order to ensure the proper operation of certain V Series modems operating at data signalling rates greater than 4800 kbit/s, it is necessary to specify improved and/or modified values for the following transmission system characteristics: random circuit noise, quantizing noise, harmonic distortion (intermodulation distortion). This subject requires further study.

4	)	Additionally, the characteristics for short breaks in transmission, phase hits, amplitude hits and low-frequency phase jitter are under study.

5	)	It is expected that in most cases these “basic bandwidth” characteristics may be available without the addition of loss/frequency and/or group-delay equalization equipment.

6	)	The application of § 3 of this Recommendation to multiterminal leased circuits is intended only for radial networks in which these specifications are to be met between a designated central station and each of the outlying stations. It does not apply to multiterminal conference networks between any two stations.

7)	The contents of § 3 correspond to part of Recommendation M.1020 [6].