|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | H.53 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  |

SERIES H: TRANSMISSION OF NON-TELEPHONE SIGNALS

Characteristics of data signals

Transmission of wide-spectrum signals
(data, etc.) over wideband supergroup links

ITU-T  Recommendation  H.53

Extract from Red Book  Fascicle III.4 (1984)

NOTES

1	ITU-T Recommendation H.53 was published in Fascicle III.4 of the Red Book. This file is an extract from the Red Book. While the presentation and layout of the text might be slightly different from the Red Book version, the contents of the file are identical to the Red Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

  ITU  1984, 1988, 1993, 1996

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation H.53

Fascicle III.4 – Rec. H.53

TRANSMISSION OF WIDE-SPECTRUM SIGNALS (DATA, ETC.)
OVER WIDEBAND SUPERGROUP LINKS

(Mar del Plata, 1968; amended at Geneva, 1972, 1976 and 1980)

Links meeting the provisions of Recommendation H.15 should be used.

# 1	Power level

1.1	The mean power level of the wideband signal over the range 312-552 kHz should not exceed _15    10 log10 60    3 dBm0.

1.2	In order to limit cross-modulation effects in wideband systems, the power level of any individual spectral component in the band 312-552 kHz should not exceed –10 dBm0, except for spectral components which are at multiples of 4 kHz, (see the Recommendation cited in [1]).

With regard to its effect on non-telephone type signals, a discrete component is defined as a signal of sinusoidal form with a minimum duration of about 100 ms.

1.3	In addition to § 1.2 above, the energy spectrum transmitted in the neighbourhood of the pilot frequencies should be limited in accordance with the Recommendation cited in [2].

# 2	Limitation of the power spectrum outside the band 312-552 kHz

The power level produced by the terminal equipment connected to the wideband supergroup link shall not exceed –73 dBm0p in any 4-kHz band outside the range 304-560 kHz.

If the terminal equipment itself does not meet these conditions, an additional filter must be applied before the point of connection to the leased supergroup link.

References

[1]	CCITT Recommendation Overall recommendations relating to carrier-transmission systems, Vol. III, Rec. G.221, § 2.2.

[2]	CCITT Recommendation Pilots on groups, supergroups, etc., Vol. III, Rec. G.241, § 7.