|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | H.263 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | Appendix I (03/97) |

SERIES H: AUDIOVISUAL AND MULTIMEDIA SYSTEMS

Infrastructure of audiovisual services – Coding of moving video

Video coding for low bit rate communication

Appendix I: Error tracking

ITU-T  Recommendation  H.263 – Appendix I

(Previously  CCITT  Recommendation)

ITU-T  H-SERIES  RECOMMENDATIONS

AUDIOVISUAL AND MULTIMEDIA SYSTEMS

| For further details, please refer to ITU-T List of Recommendations. |  |
| --- | --- |
| Characteristics of transmission channels used for other than telephone purposes | H.10–H.19 |
| Use of telephone-type circuits for voice-frequency telegraphy | H.20–H.29 |
| Telephone circuits or cables used for various types of telegraph transmission or simultaneous transmission | H.30–H.39 |
| Telephone-type circuits used for facsimile telegraphy | H.40–H.49 |
| Characteristics of data signals | H.50–H.99 |
| CHARACTERISTICS OF VISUAL TELEPHONE SYSTEMS | H.100–H.199 |
| INFRASTRUCTURE OF AUDIOVISUAL SERVICES | H.200–H.399 |
| General | H.200–H.219 |
| Transmission multiplexing and synchronization | H.220–H.229 |
| Systems aspects | H.230–H.239 |
| Communication procedures | H.240–H.259 |
| Coding of moving video | H.260–H.279 |
| Related systems aspects | H.280–H.299 |
| Systems and terminal equipment for audiovisual services | H.300–H.399 |

| ITU-T RECOMMENDATIONS SERIES |  |
| --- | --- |
| Series A | Organization of the work of the ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | Maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communication |
| Series Z | Programming languages |

*10876*

Printed in Switzerland

Geneva, 1997

FOREWORD

The ITU-T (Telecommunication Standardization Sector) is a permanent organ of the International Telecommunication Union (ITU). The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Geneva, October 1996).

Appendix I to ITU-T Recommendation H.263 was prepared by ITU-T Study Group 16 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on the 27th of March 1997.

___________________

NOTE

In this Appendix, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

  ITU  1997

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

I.1	Introduction

I.2	Error tracking

Recommendation H.263

Recommendation H.263 – Appendix I     (03/97)

VIDEO  CODING  FOR  LOW  BIT  RATE  COMMUNICATION

Appendix  I

error tracking

(Geneva, 1997)

## I.1	Introduction

This Appendix describes a method to recover efficiently after transmission errors if erroneous MBs are reported via a feedback channel to the encoder. The capability of sending and processing feedback information is signalled via external means (for example, by Recommendation H.245). Furthermore, format and content of the feedback message are defined externally (for example, by Recommendation H.245).

## I.2	Error tracking

Because INTRA coding stops temporal error propagation, it should be used for macroblocks which are severely affected by transmission errors. This requires that the location and extent of image artefacts can be made available to the encoder. The following algorithm provides an estimated error distribution based on feedback information received by the encoder. It considers spatial error propagation caused by motion-compensated prediction as well as the delay until the reception of the feedback message. The algorithm illustrates one possible approach to evaluate feedback messages for spatio-temporal error tracking. Other algorithms are possible.

Assume N macroblocks within each frame enumerated mb = 1...N from top-left to bottom-right. Let {nerr, mbfirst, mblast} be the feedback message to the encoder, where mbfirst ó mb ó mblast indicates a set of erroneous macroblocks in frame nerr.

To evaluate the feedback message, the encoder must continuously record information during the encoding of each frame. First, the initial error E0(mb, n) that would be introduced by the loss of macroblock mb in frame n needs to be stored. Assuming a simple error concealment where erroneous macroblocks are treated as not coded, E0(mb, n) is computed as the Summed Absolute Difference (SAD) of macroblock mb in frame n and n – 1. Second, the number of pixels transferred from macroblock mbsource in frame n – 1 to macroblock mbdest in frame n is stored in dependencies d(mbsource, mbdest, n). These dependencies are derived from the motion vectors.

Assume that a feedback message arrives before frame nnext is encoded, such that nnext > nerr. Then, the estimated error E(mb, nerr) in macroblock mb and frame nerr is initialized as:

E(mb,nerr)=E0(mb,nerr)0formbfirst≤mb≤mblastelse

For subsequent frames n, with nerr < n < nnext, the error may be estimated as:

E(mb,n)=i=1NE(i,n±1)d(i,mb,n)256

where a uniformly distributed error in each macroblock is assumed after each iteration.

The estimated error E(mb, nnext – 1) is incorporated into the mode decision of the next frame. For example, macroblock mb is coded in INTRA mode, if E(mb, nnext – 1) exceeds a threshold.

In practice, error tracking information will only be stored for the latest M frames. Then, if nerr < nnext – M, no error-tracking information is available and the encoder must take special action. For example, the next frame may be coded in INTRA mode. However, other procedures are possible and may be more effective.